| CSPDigital PathologyImageData StorageFormatSpecification CSP Digital Pathology Image Data Storage Format Specification |  |
| --- | --- |
| Document Version | 01 |
| Release Date | 2026-01-23 |

Pathology Branch of the Chinese Medical Association

Pathology Physicians Branch of the Chinese Medical Doctor Association

- Preface

- Digital Pathology currently lacks unified format and data standards. Data from various vendors cannot be interconnected or shared, posing obstacles to large-scale applications and hindering the full realization of pathology data value, which severely affects pathology digitization and intelligentization. The detailed issues are as follows:

- Digital Pathology does not have a unified data format standard. Each scanner outputs data in proprietary formats, and data from different vendors are incompatible and cannot be mutually accessed.

- Data sharing between applications is challenging, impacting remote pathology, regional pathology implementation, and emerging services such as artificial intelligence and big data analysis.

- Common formats exhibit considerable limitations, including incomplete record information, poor access performance, and insufficient security, which hinder the advancement of digital pathology services.

- With the support of the Pathology Branch of the Chinese Medical Association and the Pathology Physicians Branch of the Chinese Medical Doctor Association, the adoption of the pathology digital slide sharing format is promoted; the shared format uses the suffix .CSP, representing the abbreviation of Chinese Society of Pathology. This document standardizes the CSP Digital Pathology Image Data Format by integrating the characteristics of pathology services, establishing a unified format specification based on two principles: ' more comprehensive ' and ' more efficient '.

- Integrating common access patterns of pathological images with associated metadata and data improves image loading speed and optimizes the user’s slide viewing experience.

- Based on the characteristics of multi-frame pathological data, the data layout is optimized to reduce file storage space, which can be further minimized by applying secondary compression algorithms.

- Provides layout layer annotations tailored for AI algorithms, enabling simultaneous loading of images and annotation information to accommodate AI application features and effectively improve AI diagnostic efficiency.

- Enhances data verification mechanisms to ensure data security and maintain consistency of pathological files during transmission and storage.

Table of Contents

Preface	2

1 Scope	1

2 Normative Reference Documents	1

3 Terms and Definitions	1

4 Abbreviations	1

5 File Structure	2

5.1 File Structure
            	2

5.2 File Header
            	3

5.3 Data Entry
            	4

5.4 Byte Order
            	4

5.5 Data Types
            	4

5.6 Private Data Entry
            	5

6 Dataset	5

6.1 Scanner Information
            	5

6.2 Additional Image Information
            	6

6.3 Image Data
            	6

6.4 Image Metadata
            	6

6.5 Slice Information
            	7

6.6 Annotation Information
            	8

7 Data Item Extension Principles and Methods	8

7.1 Private Data Entry
            	8

7.2 Extended Types
            	8

7.3 Principles of Extension
            	8

8 Data Dictionary	8

8.1 Scanner Info Sequence
            	8

8.2 Scanner Manufacturer
            	9

8.3 Manufacturer's Model Name
            	9

8.4 Device Serial Number
            	9

8.5 Software Versions
            	9

8.6 Microns Per Pixel (MPP)
            	9

8.7 Additional Image Info Sequence
            	9

8.8 Image Type
            	9

8.9 Image Width
            	9

8.10 Image Height
            	10

8.11 Image Data Offset
            	10

8.12 Image Data Length
            	10

8.13 Barcode Value
            	10

8.14 Image Remarks
            	10

8.15 Pixel Data
            	10

8.16 Multi-Scan Result Sequence
            	10

8.17 Single Scan Result Sequence
            	11

8.18 Scan Configuration Sequence
            	11

8.19 Scan ID
            	11

8.20 Scan Time
            	11

8.21 Scan Duration
            	11

8.22 Scan Mode
            	11

8.23 Compression Algorithm
            	11

8.24 Downsampling Mode
            	12

8.25 Downsampling Ratio
            	12

8.26 Slice Standard Width
            	12

8.27 Slice Standard Height
            	12

8.28 Downsampling Width
            	12

8.29 Downsampling Height
            	12

8.30 Scan Magnification
            	12

8.31 Multi-Focal Plane Image Information Sequence
            	12

8.32 Focal Plane Information Sequence
            	13

8.33 Image ID
            	13

8.34 Number of Channels (Samples Per Pixel)
            	13

8.35 Planar Configuration
            	13

8.36 Data Representation
            	13

8.37 Image Position Z Coordinate
            	14

8.38 Image Compression Ratio
            	14

8.39 Multi-ROI Information Sequence
            	14

8.40 ROI Information Sequence
            	14

8.41 ROI Width
            	14

8.42 ROI Height
            	14

8.43 ROI X Coordinate (ROI Position X)
            	15

8.44 ROI Y Coordinate (ROI Position Y)
            	15

8.45 Multi-Downsampling Block Information Sequence
            	15

8.46 Downsampling Block Information Sequence
            	15

8.47 Downsampling Block Width
            	15

8.48 Sample Block Height
            	15

8.49 Sample Frame Number
            	15

8.50 Sample Block Position X
            	15

8.51 Sample Block Position Y
            	15

8.52 Multi-Frame Info Sequence
            	16

8.53 Frame Info Sequence
            	16

8.54 Frame ID
            	16

8.55 Frame Ratio
            	16

8.56 Frame Width
            	16

8.57 Frame Height
            	16

8.58 Multi-Tile Info Sequence
            	16

8.59 Tile Info
            	16

8.60 Specimen Info Sequence
            	17

8.61 Slide Number
            	17

8.62 Sample Type
            	17

8.63 Sample Name
            	23

8.64 Specimen Source
            	23

8.65 Material Position
            	23

8.66 Slide Type
            	32

8.67 Antibody Name
            	32

8.68 Pathology Number
            	32

8.69 Subspecialty
            	32

8.70 Patient ID
            	33

8.71 Patient Name
            	33

8.72 Patient Gender
            	33

8.73 Date of Birth
            	33

8.74 Identification Document Type
            	33

8.75 Identification Document Number
            	34

8.76 Referring Hospital
            	34

8.77 Referring Department
            	34

8.78 Submission Time
            	34

8.79 Inpatient Number
            	34

8.80 Outpatient Number
            	34

8.81 Patient Ward
            	34

8.82 Bed Number
            	34

8.83 Annotation Information Multi-Annotation Sequence
            	35

8.84 Rectangle Annotation
            	35

8.85 Position Annotation
            	35

8.86 Outline Annotation
            	35

9 References	36

Scope

This document specifies the data format requirements, data storage requirements, and compatibility requirements for CSP Digital Pathology Image data. It applies to healthcare institutions at all levels for the acquisition, storage, and application centered on Digital Pathology Images, thereby standardizing the use of pathology image data in medical institutions, reducing the storage demands of pathology image data, and improving the interoperability of pathology image data.

This standard applies to the development of the CSP Digital Pathology Image SDK, image acquisition in CSP Digital Pathology systems, CSP image storage, CSP image analysis, and related functions.

Normative Reference Documents

The contents of the following documents, referenced normatively in this text, constitute essential provisions of this document. For dated referenced documents, only the version corresponding to the specified date applies to this document; For undated referenced documents, the latest version (including all amendments) applies to this document.

GB/T 1.1-2009 Guidelines for Standardization Part 1: Structure and Drafting of Standards

GB/T 7027 Basic Principles and Methods of Information Classification and Coding

Terms and Definitions

The following terms and definitions apply to this document.

3.1  Digital Pathology

Digital Pathology refers to the application of computers and networks within the pathology field, where high-resolution digital images are acquired through fully automated microscopes or optical magnification systems. Subsequently, computers automatically perform high-precision, multi-view seamless stitching and processing of the obtained images to produce high-quality visual data for use in complex workflows across various pathology disciplines.

3.2 Digital Pathology Image

A Digital Pathology Image is the digitalization of a physical pathological specimen, whereby the pathological slide is quickly scanned into a high-resolution digital image containing all tissue information, enabling digital pathological slide review and related operations. Its essence is to achieve comprehensive, high-definition, and standardized image acquisition; therefore, it is also known as a Whole Slide Image (WSI).

3.3 Digital Slide Scanner

The digital slide scanner is a precision instrument integrating optics, mechanics, electronics, and computer science. It acquires multiple contiguous high-resolution microscopic images by controlling the microscopic imaging system and the slide to move according to a specific pattern, then seamlessly stitches them to generate a high-resolution whole-slide digital image (WSI).

3.4 Storage Format

Refers to the method of storing computer information in a specific manner, including sound, images, etc. In this document, it specifically denotes the data arrangement format of digital pathology image files.

3.5 Region Of Interest (ROI)

An image region selected within the image, which constitutes the key focus of image analysis. This region is delineated for subsequent processing. Using an ROI to delineate the target reduces processing time and enhances accuracy.

3.6　Downsampling

A technique employed to reduce data size, typically used in digital pathology images to convert high-resolution images into lower-resolution images.

Abbreviations

The following abbreviations apply to this document.

| English Abbreviation | English Full Term | Chinese Explanation |
| --- | --- | --- |
| AI | Artificial Intelligence | Artificial Intelligence |
| GID | Group ID | Group Identifier |
| JPEG | Joint Photographic Experts Group | ISO/ITU standard for compression of static images |
| PIS | Pathology Information System | Pathology Information System |
| QoS | Quality of Service | Service Quality |
| ROI | Region of Interest | Region of Interest |
| SDK | Software Development Kit | Software Development Kit |
| UID | User Identity | User Identity Identifier |
| WSI | Whole Slide Image | Whole-slide digital image |
| MPP | Microns Per Pixel | Microns per pixel |
| CSP | Chinese Society of Pathology | Pathology Branch of the Chinese Medical Association |

File Structure

File Structure

The data format of CSP Digital Pathology Image is primarily used to describe the relevant data information of a pathological digital slide file generated by scanning a physical slide and is applied in diagnostic slide reading, artificial intelligence analysis, and related scenarios. The overall structure comprises a File header (fixed length of 128 bytes), Scanner information section, Additional Images information section, Image data section, Image metadata section, Slide information section, and Annotation information section. Except for the File header, the remaining sections consist of multiple Data Entries, as illustrated below.

File header

The File header records information as detailed in the following table, where all fields are stored in little-endian format.

| Byte | Byte Length | Field Name | Field Description | Remarks |
| --- | --- | --- | --- | --- |
| 1-8 | 8 | signature | This is used to identify the file as a CSP Digital Pathology Image file format. | The valid value is " MEDIC" |
| 9-12 | 4 | version | Version Number |  |
| 13-14 | 2 | offset type | Addressing mode representing an offset. 1 6: 16-bit addressing 3 2: 32-bit addressing 6 4: 64-bit addressing | If the value is 3 2, then OFF_TYPE denotes a uint32 type, with OFF_LEN=4; If the value is 6 4, then OFF_TYPE denotes a uint64 type, with OFF_LEN=8. |
| 15-30 | 16 | protocol | Indicates the protocol type utilized by this file | Valid value is " STANDARD" |
| 31-38 | 8 | multi scan result offset | Indicates the file offset of multiple scan results |  |
| 39-40 | 2 | string encode | String Encoding Method | Valid values are as follows: 1: Denotes UTF-8 Note: This field specifies the encoding method for all strings within the file |
| 41-42 | 2 | security | File Confidentiality Level | Valid values are as follows: 1: Indicates External Disclosure 2: Indicates Internal Disclosure 3: Indicates Confidential 4: Indicates Top Secret |
| 43-128 | - | reserved | reserved | Reserved field |

Data Entry

Data Entry describes all information except the File header, with its structure illustrated in the figure below:

Each Data Entry comprises the following fields:

| Unique ID (UID) | Module ID (MDID) | The module ID of the Data Entry module, with a length of 2 bytes. |
| --- | --- | --- |
|  | Entry ID (EID) | The sequence ID of the Data Entry within the module, with a length of 2 bytes. |
| Data Type (DT) |  | Data format used to identify the Value field of a Data Entry, with a length of 2 bytes. |
| Value Num (VN) |  | Indicates the number of values in the Value field, with a length of OFF_LEN bytes. |
| Value Length (VL) |  | Indicates the byte length of the Value field, with a length of OFF_LEN bytes. The value of the Value Length must be even. If the actual byte length of the value is odd, the value shall be padded according to the following rules: DT value is STRING or TEXT: Pad the end of the value domain with space characters, i.e., 20H. DT is other values: Pad the end of the value domain with NULL characters, i.e., 00H. |
| Value |  | Value field containing bytes of Value Length |

Definitions of Data Entries with different UIDs are detailed in Chapter 8 'Data Dictionary'.

Byte order

Unless otherwise specified, all data in Data Entry is stored in little-endian format.

Data type

The DT field in Data Entry denotes the Data type of the Value field. The following table lists all possible DT values and their descriptions:

| DT Name | DT Value | DT Description |
| --- | --- | --- |
| BYTE | 0x0001 | 8-bit unsigned integer (8-bit unsigned integer) |
| SBYTE | 0x0002 | 8-bit signed integer (8-bit signed integer) |
| SHORT | 0x0003 | 16-bit unsigned integer (1 6-bit unsigned integer) |
| SSHORT | 0x0004 | 16-bit signed integer (1 6-bit signed integer) |
| LONG | 0x0005 | 32-bit unsigned integer (3 2-bit unsigned integer) |
| SLONG | 0x0006 | 32-bit signed integer (3 2-bit signed integer) |
| LONG8 | 0x0007 | 64-bit unsigned integer (6 4-bit unsigned integer) |
| SLONG8 | 0x0008 | 64-bit signed integer (6 4-bit signed integer) |
| FP32 | 0x0009 | 32-bit single-precision floating-point (32-bit single-precision floating-point), using the IEEE754 floating-point standard. |
| FP64 | 0x000a | 6 4-bit double-precision floating-point (6 4-bit double-precision floating-point), conforming to the IEEE 754 floating-point standard. |
| RATIONAL | 0x000b | Rational numbers consist of two S LONG values, specifically an R ATIONAL of 8 bytes in length, where every four bytes represent one S LONG ; the first S LONG indicates the numerator, and the second S LONG indicates the denominator. |
| STRING | 0x000c | Short string with a length less than or equal to 2 55 bytes |
| TEXT | 0x000d | Long string, length less than or equal to 2 32-2 bytes |
| SEQUENCE | 0x000e | Sequence format indicating that the value domain of this Data Entry is a sequence of Data Entries, with support for extension. |
| UNDEFINED | 0x000f | Non-standard defined encoding format, with parsing methods specified by the specific U ID |

Private Data Entry

Data Entries with EID ranges from 0xF000 to 0xFFFF within standardized MDIDs, as well as Data Entries with any EID within non-standardized MDIDs, are designated for user-defined Private Data Entries.

Dataset

This chapter defines the dataset contents of the CSP Digital Pathology Image file. According to the byte order in the file’s data region, it is divided into several parts: Scanner information, Additional Images information, Image data, Image metadata, Slide information, and Annotation information. Each part is represented by a Data Entry structure. The format standard categorizes all Data Entries modularly based on their functional roles, as outlined in the following module information:

| module ID | module name | description |
| --- | --- | --- |
| 0x0001 | Scanner | Scanner information |
| 0x0002 | Whole Slide Image | Image information |
| 0x0003 | Image Pixel | Pixel Data |
| 0x0004 | Scan Configuration | Scan configuration information |
| 0x0005 | Scan Result | Scan Results |
| 0x0006 | Down Sample | Downsampling Information |
| 0x0007 | Specimen | Sample Information |
| 0x0008 | Patient | Patient Information |
| 0x0009 | Annotation | Annotation Information |

Scanner information

Scanner information primarily comprises the hardware and software details involved in scanning the pathological section, as well as information concerning the scanning operation itself, including the following fields:

| module name | entry name | description |
| --- | --- | --- |
| Scanner | Manufacture | Scanner Manufacturer |
| Scanner | Manufacture's Model Name | Device Model |
| Scanner | Device Serial Number | Device Serial Number |
| Scanner | Software Versions | Software Version |
| Scanner | Microns Per Pixel (MPP) | Microns per pixel |

Additional Images Information

A CSP Digital Pathology Image file may contain multiple Additional Images used to represent preview images, Label Maps, thumbnails, and the like. Each Additional Image is described by an Additional Images Information Data Entry, which typically includes the following fields:

| module name | entry name | description |
| --- | --- | --- |
| Whole Slide Image | Image Type | Image Type |
| Whole Slide Image | Image Width | Image Height |
| Whole Slide Image | Image Height | Image Width |
| Whole Slide Image | Image Data Offset | Image data offset |
| Whole Slide Image | Image Data Length | Image data length |
| Whole Slide Image | Barcode Value | Barcode |
| Whole Slide Image | Image Remarks | Remark Information |

When Additional Images represent Label Maps, they may also include the Barcode corresponding to the label and associated remark information.

Image data

The Image Data Data Entry contains all pixel data of the entire file, including pixel data of all Additional Images as well as the pixel data generated from each scan.

Image metadata

A CSP Digital Pathology Image file can represent multiple pathology images, which are the scanning results of the same Slide obtained through different scanning methods.

Image metadata is represented by a Multi-scan Results Data Entry, which contains several Single-scan Results Data Entries. For each Single-scan Result, the Scan Configuration Information Data Entry and Multi-image Information Data Entry are recorded as illustrated in the following diagram:

Among them, one Multi-image information represents a complete pathological scan result, which may correspond to a brightfield pathological scan at a specific layer or a fluorescence pathological scan of a particular channel. One Multi-image information Data Entry contains several Image information Data Entries, each representing a scanned image of a single fixed focal plane. Each Image information Data Entry stores image data at various magnification levels using the traditional pyramid model. Scan configuration information includes the following fields:

| module name | entry name | description |
| --- | --- | --- |
| Scan Configuration | Scan ID | Scan serial number |
| Scan Configuration | Scan Time | Scan timestamp |
| Scan Configuration | Scan Duration | Scan duration |
| Scan Configuration | Scan Mode | Scan mode |
| Scan Configuration | Compress Method | Compression algorithm |
| Scan Configuration | Down Sampling Mode | Downsampling Mode |
| Scan Configuration | Down Sampling Ratio | Downsampling Factor |
| Scan Configuration | Slice Basic Width | Standard Block Width |
| Scan Configuration | Slice Basic Height | Standard Block Height |
| Scan Configuration | Down Sampling Width | Standard downsampled width |
| Scan Configuration | Down Sampling Height | Standard downsampled height |
| Scan Configuration | Scan Ratio | Scan magnification |
| Whole Slide Image | Image Type | Image Type |

Slide information

Represents the basic medical information of the pathological section, including information about the sample itself and relevant patient information; it comprises the following fields:

| module name | entry name | description |
| --- | --- | --- |
| Specimen | Slide No | Sample Serial Number |
| Specimen | Sample Type | Specimen Type |
| Specimen | Sample Name | Specimen Name |
| Specimen | Material Position | Sampling site |
| Specimen | Slide Type | Slide Type |
| Specimen | Antibody Name | Antibody name |
| Specimen | Pathology No | Pathology Number |
| Patient | Subspecialty | Subspecialty |
| Patient | Patient ID | Patient I D |
| Patient | Patient Name | Patient name |
| Patient | Patient Sex | Patient sex |
| Patient | Date of Birth | Date of birth |
| Patient | Card Type | Identification Type |
| Patient | Card No | Identification Number |
| Patient | Send Hospital | Submitting Institution |
| Patient | Send Department | Submitting Department |
| Patient | Send Time | Specimen Submission Time |
| Patient | Inpatient No | Hospitalization Number |
| Patient | Outpatient No | Outpatient Number |
| Patient | Patient Area | Ward |
| Patient | Bed No | Bed Number |

Annotation Information

Used to describe the annotation information of this file, including manual and AI annotations.

Principles and Methods of Data Element Extension

Private Data Entry

Data Entries with EID ranging from 0xF000 to 0xFFFF within the specified MDID, as well as Data Entries with any EID within unspecified MDIDs, are designated for user-defined Private Data Entries.

Types of Extensions

The following types of extensions are permitted:

a) Addition of new data elements;

b) Addition of new data item entities;

c) Imposition of more stringent optional constraints on existing data elements;

Principles of Extension

The requirements for implementing data element extension are as follows:

a) Extended data elements shall not share the same name or meaning as those already defined in this standard;

b) Extended data items may be defined as entities, which can include both extended and existing data elements as their components;

c) For each extended data item entity and element, the Chinese name, English name, definition, data type, and examples shall be provided.

Data Dictionary

Scanner Information Scanner Info Sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0001 | 0x0001 | SEQUENCE | 0-n |

Document the hardware and software information during the scanning of the pathological section.

Scanner Manufacturer Manufacture

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0001 | 0x0002 | STRING | 1 |

Record the manufacturer of the scanner that produced the Slide, with a maximum length of 64 bytes.

Device Model M anufacture's Model Name

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0001 | 0x0003 | STRING | 1 |

Used to record the device model of the scanner that produced the Slide, with a length not exceeding 64 bytes.

设备序列号Device Serial Number

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0001 | 0x0004 | STRING | 1 |

Used to record the serial number of the scanner device that produced the Slide, with a length not exceeding 64 bytes.

软件版本Software Versions

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0001 | 0x0005 | STRING | 1 |

Used to record the software version of the scanner that produced the Slide, with a length not exceeding 64 bytes.

Microns per pixel M PP

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0001 | 0x0005 | FLOAT | 1 |

Used to specify the resolution of the slide, for example, 0.25 microns per pixel.

Additional Images Additional Image Info Sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0001 | SEQUENCE | 5-n |

Used to record the preview image, Label Map, and thumbnail of the corresponding slide image; a single file may contain multiple Additional Images.

Image Type Image Type

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0002 | BYTE | 1 |

Indicates the image type corresponding to the Additional Images, which includes the following categories:

0: Represents the Label Map

1: Represents the Preview Image

2: Represents the Thumbnail

Image Width Image Width

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0003 | LONG | 1 |

Indicates the image width of the Additional Images.

Image Height Image Height

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0004 | LONG | 1 |

Indicates the image height of the Additional Images.

Image data offset Image Data Offset

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0005 | LONG8 | 1 |

Used to indicate the relative offset of the Additional Images’ pixel data within the value domain of the Image data.

Image data length Image Data Length

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0006 | LONG8 | 1 |

Used to indicate the byte length of pixel data for the Additional Images.

Barcode Barcode Value

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0007 | STRING | 1 |

This field only exists in the sub-items of Additional Images and solely when the Additional Image type specifies a Label Map; it is used to indicate the barcode value of the corresponding Label Map.

Image remarks

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0008 | TEXT | 1 |

Used to record the annotation information of the Additional Images.

Image data Pixel Data

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0003 | 0x0001 | BYTE | n |

Used to record all pixel data throughout the entire file.

Assuming a file contains a preview image, a Label Map, a thumbnail, and a scan result, the data arrangement of the value domain within the image data may be as follows:

Accordingly, the image data offset for the preview image is 0, for the Label Map is len1, and so forth.

多次扫描结果Multi Scan Result Sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0005 | 0x0001 | SEQUENCE | 1-n |

Used to record metadata corresponding to multiple scans of a single slide, with the value containing at least one single scan result structure.

Single Scan Result S can Result Sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0005 | 0x0002 | SEQUENCE | 2-n |

Used to represent the result of a single scan, comprising one Scan configuration information Data Entry and one Multi-image information Data Entry.

Scan configuration sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0004 | 0x0001 | SEQUENCE | 0-n |

Used to represent the configuration information corresponding to the scan.

Scan serial number

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0004 | 0x0002 | LONG | 1 |

Used to represent the scan serial number corresponding to the scan; the scan serial number must be unique for multiple scans.

Scan timestamp

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0004 | 0x0003 | STRING | 1 |

Used to represent the start time of the scan.

Scan duration

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0004 | 0x0004 | LONG | 1 |

Used to represent the duration of the scan, in milliseconds (ms).

Scan mode

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0004 | 0x0005 | BYTE | 1 |

Used to represent the scan mode corresponding to the scan, including linear array mode and area array mode.

Compression Algorithm C ompress Method

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0004 | 0x0006 | BYTE | 1 |

Indicates the compression algorithm used to store the pixel data generated by the scan.

0: Indicates uncompressed data

5: Indicates LZW compression

8: Indicates deflate compression

12: Indicates JPEG compression

13: Indicates JPEG2000 compression

Down Sampling Mode D own Sampling Mode

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0006 | 0x0001 | BYTE | 1 |

Indicates the sampling mode applied during down sampling of the pixel data generated by the scan.

Down Sampling Ratio D own Sampling Ratio

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0006 | 0x0002 | FLOAT | 1 |

Indicates the scaling ratio applied during down sampling of the scan data.

Slice Basic Width S lice Basic Width

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0004 | 0x0007 | LONG | 1 |

Indicates the standard width used for slicing the scan data.

切块标准高度Slice Basic Height

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0004 | 0x0008 | LONG | 1 |

Indicates the standard height of the slice for the current scan data.

下采样标准宽度Down Sampling Width

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0006 | 0x0003 | LONG | 1 |

Indicates the standard width of the downsampled block during downsampling of the current scan data.

下采样标准高度Down Sampling Height

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0006 | 0x0004 | LONG | 1 |

Indicates the standard height of the downsampled block during downsampling of the current scan data.

扫描倍率Scan Ratio

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0004 | 0x0009 | FLOAT | 1 |

Used to indicate the magnification of the slide in this scan.

Multi-image information M ulti Focal Plane Sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0009 | SEQUENCE | 1-n |

Used to represent all image data generated by this scan.

Image information F ocal Plane Info Sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x000a | SEQUENCE | 0-n |

Represents the pyramid image corresponding to a single focal plane.

Image identifier Image ID

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x000b | LONG | 1 |

Used to indicate the Image identifier corresponding to the image, which must be unique within a single file.

Number of Channels Samples Per Pixel

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x000c | LONG | 1 |

Used to indicate the Number of Channels corresponding to the image color, such as 3 or 4 channels for JPEG images.

Plane configurationP lanar Configuration

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x000d | BYTE | 1 |

This field exists only when the value of Samples Per Pixel is greater than 1 and is used to specify the data storage method for multi-channel images.

Valid values are:

1: BI mode, in which different channels of the same pixel are stored together.

2: BSQ mode, in which data of the same channel are stored together, while data of adjacent channels are stored consecutively.

Note:

The following illustrates data storage examples for 3-channel data in both BI mode and BSQ mode:

This field is irrelevant in compressed images.

Data typeData Representation

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x000e | BYTE | 1 |

Data type used to represent the basic data units of image pixels.

1：uint8_t

2：int8_t

3：uint16_t

4：int16_t

5：uint32_t

6：int32_t

7：uint64_t

8：int64_t

9：float

10：double

Image Z-coordinate Image Position Z

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x000f | LONG | 1 |

Used to denote the Z-axis coordinate value of the focal plane corresponding to the image.

Image compression ratio Image Compress Ratio

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0010 | FP32 | 1 |

Represents the compression ratio of the image data during compression.

Compression ratio = Original image data byte length / Compressed data byte length

Multi R OI Information Mul ti ROI Info Sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0011 | SEQUENCE | 0-n |

Used to represent the ROI region information within the image.

ROI Information R OI Info Sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0012 | SEQUENCE | 4-n |

Used to represent information of a single ROI region.

ROI Width R OI Width

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0013 | LONG | 1 |

Used to denote the width of the ROI region.

ROI Height ROI Height

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0014 | LONG | 1 |

Indicates the height of the ROI region.

ROI Coordinate X ROI Position X

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0015 | LONG | 1 |

Represents the starting coordinate X of the ROI region.

ROI Coordinate Y ROI Position Y

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0016 | LONG | 1 |

Represents the starting coordinate Y of the ROI region.

多下采样块信息Multi Sample Blocks Sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0017 | SEQUENCE | 0-n |

Used to represent block information during image downsampling.

Downsampling Block Information Sample Block Info Sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0018 | SEQUENCE | 0-n |

Used to represent the information of a single downsampled block.

Downsampled block width Sample Block Width

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0019 | LONG | 1 |

Used to indicate the actual width of the downsampled block.

Downsampled block heightS ample Block Height

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x001a | LONG | 1 |

Indicates the actual height of the downsampled block.

Number of downsampled frames Sample Frame Num

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x001b | LONG | 1 |

Used to indicate the total number of frames in the downsampled block.

Downsampled block X coordinate Sample Block Position X

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x001c | LONG | 1 |

Represents the starting coordinate X of the downsampled block.

Downsampled block Y coordinate S ample Block Position Y

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x001d | LONG | 1 |

Indicates the starting Y coordinate of the downsampling block.

Multi-frame informationM ulti Frame Info Sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x001e | SEQUENCE | 1-n |

Used to represent the frame information for all frames within this pyramid image.

Frame informationF rame Info Sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x001f | SEQUENCE | 5-n |

Used to represent the information of a single frame in the pyramid image.

Frame identifier F rame ID

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0020 | LONG | 1 |

Used to indicate the Frame identifier of the frame. The Frame identifier should increment sequentially by 1 from the bottom layer of the pyramid upward, with the Frame identifier of the bottom-most frame set to 0.

Frame magnification Frame Ratio

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0021 | FLOAT | 1 |

Used to represent the scaling factor of the frame.

Frame width F frame width

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0022 | LONG | 1 |

Used to represent the width of the frame.

Frame heightF Frame Height

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0023 | LONG | 1 |

Used to represent the height of the frame.

多图像块信息Multi Tile Info Sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0024 | SEQUENCE | 0-n |

Used to represent all image block information of the frame.

Image Block Information Tile Info

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0002 | 0x0025 | UNDEFINED | 1 |

Used to represent the information of a single image tile. The specific definition of its value domain is as follows:

| Byte | Byte Length | Data type | Field Name | Field Description | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1-4 | 4 | uint32 | tile width | Width of the image block |  |
| 5-8 | 4 | uint32 | tile height | Height of the image block |  |
| 9-16 | 8 | uint64 | tile data offset | Data Offset of the Image Block |  |
| 17-24 | 8 | uint64 | tile data length | Data length of the image tile |  |
| 25-28 | 4 | uint32 | tile position X | X coordinate of the image tile |  |
| 29-32 | 4 | uint32 | tile position Y | Y-coordinate of the image tile |  |
| 33-36 | 4 | uint32 | tile data CRC32 | Image Tile Data CRC32 Checksum |  |

Slide information Specimen Info Sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0007 | 0x0001 | SEQUENCE | 1 |

Used to represent all pathological information of the slide.

Slide Number S lide No

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0007 | 0x0002 | STRING | 1 |

Used to specify the slide number, with a length not exceeding 8 bytes.

Specimen Type S ample Type

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0007 | 0x0003 | SHORT | 1 |

DT is used to represent the specimen type, denoted by the <System, Specimen, Specimen Type> triplet, occupying 8 bits, 12 bits, and 12 bits respectively in LONG data, as illustrated in the figure below:

The enumeration definitions are listed in the table below:

| System Name | System ID | Specimen Name | Specimen ID | Specimen Type | Specimen Type Index |
| --- | --- | --- | --- | --- | --- |
| Head and Neck | 0x01 | Thyroid and Parathyroid | 0x01 | Total Thyroidectomy Specimen | 0x01 |
|  |  |  |  | Subtotal Thyroidectomy Specimen | 0x02 |
|  |  |  |  | Partial thyroidectomy specimen | 0x03 |
|  |  |  |  | Thyroid and lymph node excision specimen | 0x04 |
|  |  |  |  | Thyroid puncture biopsy specimen | 0x05 |
|  |  |  |  | Parathyroidectomy Specimen | 0x06 |
|  |  | Hypopharynx (laryngopharynx), larynx, trachea, and parapharyngeal space | 0x02 | Total laryngectomy specimen | 0x01 |
|  |  |  |  | Subtotal laryngectomy specimen | 0x02 |
|  |  |  |  | Laryngeal tumor resection specimen | 0x03 |
|  |  |  |  | Laryngoscopic biopsy specimen | 0x04 |
|  |  | Nasopharynx | 0x03 | Nasopharyngeal biopsy specimen | 0x01 |
|  |  |  |  | Nasopharyngeal tumor resection specimen | 0x02 |
|  |  |  |  | Nasal endoscopic biopsy specimen | 0x03 |
|  |  |  |  | Nasal endoscopic resection specimen | 0x04 |
|  |  | Nasal cavity, paranasal sinuses, and skull base | 0x04 | Nasal cavity biopsy specimen | 0x01 |
|  |  |  |  | Nasal cavity tumor resection specimen | 0x02 |
|  |  |  |  | Nasal endoscopic biopsy specimen | 0x03 |
|  |  |  |  | Partial maxillectomy specimen | 0x04 |
|  |  |  |  | Extended maxillectomy specimen | 0x05 |
|  |  | Salivary gland | 0x05 | Salivary gland total excision specimen | 0x01 |
|  |  |  |  | Salivary gland partial specimen | 0x02 |
|  |  |  |  | Salivary gland mass excision specimen | 0x03 |
|  |  |  |  | Salivary gland mass (puncture) biopsy specimen | 0x04 |
|  |  | Oropharynx (base of tongue, tonsil, adenoid) | 0x06 | Base of tongue mass biopsy specimen | 0x01 |
|  |  |  |  | Tonsil excision specimen | 0x02 |
|  |  |  |  | Adenoid excision specimen | 0x03 |
|  |  | Oral cavity (lip) and mobile tongue | 0x07 | Oral mucosal biopsy specimen | 0x01 |
|  |  |  |  | Glossectomy | 0x02 |
|  |  | Odontogenic and maxillofacial bone tumors | 0x08 | Maxillectomy | 0x01 |
|  |  |  |  | Mandibulectomy | 0x02 |
|  |  | Ear | 0x09 | Ear canal biopsy specimen | 0x01 |
|  |  |  |  | Ear canal mass excision specimen | 0x02 |
|  |  | Eye | 0x0a | Resected Specimen | 0x01 |
|  |  |  |  | Biopsy Specimen | 0x02 |
| Respiratory System | 0x02 | Lung | 0x01 | Wedge Resection Lung Specimen | 0x01 |
|  |  |  |  | Segmental Lung Resection Specimen | 0x02 |
|  |  |  |  | Lobar Lung Resection Specimen | 0x03 |
|  |  |  |  | Bilateral Lobar Lung Resection Specimen | 0x04 |
|  |  |  |  | Pneumonectomy Specimen | 0x05 |
|  |  |  |  | Bronchoscopic Biopsy Specimen | 0x06 |
|  |  |  |  | Percutaneous Lung Needle Biopsy Specimen | 0x07 |
|  |  |  |  | Lung Tissue Biopsy Specimen | 0x08 |
| Pleural Tumor | 0x03 | Pleura | 0x01 | Fine Needle Aspiration Biopsy Specimen | 0x01 |
|  |  |  |  | Open Biopsy Specimen | 0x02 |
|  |  |  |  | Thoracoscopic Biopsy Specimen | 0x03 |
|  |  |  |  | Extrapleural Fibrolysis Specimen | 0x04 |
|  |  |  |  | Radical pleurectomy specimen | 0x05 |
|  |  |  |  | Extrapleural pneumonectomy specimen | 0x06 |
|  |  |  |  | Cytoreductive surgery specimen | 0x07 |
| Digestive system | 0x04 | Esophagus | 0x01 | Endoscopic biopsy specimen | 0x01 |
|  |  |  |  | Endoscopic submucosal dissection specimen (ESD) | 0x02 |
|  |  |  |  | Endoscopic mucosal resection specimen (EMR) | 0x03 |
|  |  |  |  | Partial esophagectomy specimen | 0x04 |
|  |  |  |  | Partial esophagogastrectomy specimen | 0x05 |
|  |  | Stomach | 0x02 | Endoscopic biopsy specimen | 0x01 |
|  |  |  |  | Endoscopic submucosal dissection specimen (ESD) | 0x02 |
|  |  |  |  | Endoscopic mucosal resection specimen (EMR) | 0x03 |
|  |  |  |  | Proximal gastrectomy specimen | 0x04 |
|  |  |  |  | Distal gastrectomy specimen | 0x05 |
|  |  |  |  | Total gastrectomy specimen | 0x06 |
|  |  |  |  | Partial gastrectomy specimen | 0x07 |
|  |  |  |  | Partial esophagogastrectomy specimen | 0x08 |
|  |  | Small intestine | 0x03 | Endoscopic biopsy specimen | 0x01 |
|  |  |  |  | Polyp resection specimen | 0x02 |
|  |  |  |  | Whipple procedure resection specimen | 0x03 |
|  |  |  |  | Partial small intestine resection specimen | 0x04 |
|  |  |  |  | Small intestine resection specimen | 0x05 |
|  |  |  |  | Pancreaticoduodenectomy specimen | 0x06 |
|  |  |  |  | Partial or total small intestine resection specimen | 0x07 |
|  |  | Colorectum | 0x04 | Resected Specimen | 0x01 |
|  |  |  |  | Biopsy Specimen | 0x02 |
|  |  | Liver (including intrahepatic bile ducts) | 0x05 | Liver core needle biopsy specimen | 0x01 |
|  |  |  |  | Liver wedge resection specimen | 0x02 |
|  |  |  |  | Partial liver resection specimen | 0x03 |
|  |  |  |  | Liver lobectomy specimen | 0x04 |
|  |  |  |  | Total liver resection specimen | 0x05 |
|  |  | Gallbladder and extrahepatic bile ducts | 0x06 | Endoscopic biopsy specimen | 0x01 |
|  |  |  |  | Cholecystectomy specimen | 0x02 |
|  |  |  |  | Extrahepatic bile duct resection specimen | 0x03 |
|  |  |  |  | Local or segmental bile duct resection specimen | 0x04 |
|  |  | Pancreas | 0x07 | Pancreatic Puncture Biopsy Specimen | 0x01 |
|  |  |  |  | Pancreaticoduodenectomy (Whipple Procedure) Specimen | 0x02 |
|  |  |  |  | Partial Pancreatectomy Specimen | 0x03 |
|  |  |  |  | Pylorus-Preserving Pancreaticoduodenectomy Specimen | 0x04 |
| Breast | 0x05 | Breast | 0x01 | Total Mastectomy (Including Axillary Lymph Nodes/Sentinel Lymph Nodes) Specimen | 0x01 |
|  |  |  |  | Partial Mastectomy Specimen | 0x02 |
|  |  |  |  | Breast Mass Excision Specimen | 0x03 |
|  |  |  |  | Breast Puncture Biopsy Specimen | 0x04 |
| Female Reproductive System | 0x06 | Vulva | 0x01 | Vulvar Biopsy Specimen | 0x01 |
|  |  |  |  | Vulvectomy Specimen | 0x02 |
|  |  |  |  | Radical Vulvectomy Specimen | 0x03 |
|  |  | Vagina | 0x02 | Vaginal biopsy specimen | 0x01 |
|  |  |  |  | Vaginal excision specimen | 0x02 |
|  |  | Cervix | 0x03 | Cervical biopsy specimen | 0x01 |
|  |  |  |  | Cold knife conization cervical specimen | 0x02 |
|  |  |  |  | Cervical LEEP excision specimen | 0x03 |
|  |  |  |  | Cervical excision specimen | 0x04 |
|  |  |  |  | Whole uterus excision specimen | 0x05 |
|  |  |  |  | Whole uterus, bilateral ovaries, fallopian tubes, and pelvic lymph nodes excision specimen | 0x06 |
|  |  |  |  | Whole uterus, left ovary, left fallopian tube, and pelvic lymph nodes excision specimen | 0x07 |
|  |  |  |  | Whole uterus, right ovary, right fallopian tube, and pelvic lymph nodes excision specimen | 0x08 |
|  |  |  |  | Pelvic organs en bloc excision specimen | 0x09 |
|  |  | Uterine body | 0x04 | Endometrial curettage specimen | 0x01 |
|  |  |  |  | Uterine fibroid resection specimen | 0x02 |
|  |  |  |  | Whole uterus excision specimen | 0x03 |
|  |  |  |  | Whole uterus and bilateral adnexa resection specimen | 0x04 |
|  |  |  |  | Whole uterus, bilateral adnexa, and pelvic lymph nodes resection specimen | 0x05 |
|  |  |  |  | Pelvic organs en bloc excision specimen | 0x06 |
|  |  | Gestational trophoblastic tumor | 0x05 | Endometrial curettage specimen | 0x01 |
|  |  |  |  | Whole uterus excision specimen | 0x02 |
|  |  |  |  | Whole uterus and bilateral adnexa resection specimen | 0x03 |
|  |  |  |  | Whole uterus, bilateral adnexa, and pelvic lymph nodes resection specimen | 0x04 |
|  |  |  |  | Pelvic organs en bloc excision specimen | 0x05 |
|  |  |  |  | Placental abruption specimen | 0x06 |
|  |  | Ovary and fallopian tube | 0x06 | Ovarian resection specimen | 0x01 |
|  |  |  |  | Partial ovarian resection specimen | 0x02 |
|  |  |  |  | Fallopian tube-ovary resection specimen | 0x03 |
|  |  |  |  | Ovarian tumor resection specimen | 0x04 |
|  |  |  |  | Ovarian tumor enucleation specimen | 0x05 |
|  |  |  |  | Fallopian tube resection specimen | 0x06 |
|  |  |  |  | Partial fallopian tube resection specimen | 0x07 |
|  |  |  |  | Specimen of fallopian tube tumor resection | 0x08 |
|  |  |  |  | Specimen of whole uterus, bilateral ovaries, bilateral fallopian tubes, and pelvic lymph node resection | 0x09 |
|  |  |  |  | Whole uterus, left ovary, left fallopian tube, and pelvic lymph nodes excision specimen | 0x0a |
|  |  |  |  | Whole uterus, right ovary, right fallopian tube, and pelvic lymph nodes excision specimen | 0x0b |
|  |  |  |  | Pelvic organs en bloc excision specimen | 0x0c |
|  |  | Placenta | 0x07 | Placental abruption specimen | 0x01 |
|  |  |  |  | Placenta and partial uterus resection specimen | 0x02 |
|  |  |  |  | Placenta and whole uterus resection specimen | 0x03 |
|  |  |  |  | Endometrial curettage specimen | 0x04 |
|  |  | Peritoneum | 0x08 | Peritoneal biopsy specimen | 0x01 |
|  |  |  |  | Peritoneal resection specimen | 0x02 |
|  |  | Broad ligament and other uterine ligaments | 0x09 | Mass biopsy specimen | 0x01 |
|  |  |  |  | Mass resection specimen | 0x02 |
| Male reproductive system | 0x07 | Testis | 0x01 | Testis resection specimen | 0x01 |
|  |  |  |  | Testis and epididymis resection specimen | 0x02 |
|  |  |  |  | Testicular tumor biopsy specimen | 0x03 |
|  |  |  |  | Epididymis resection specimen | 0x04 |
|  |  |  |  | Testicular puncture biopsy specimen | 0x05 |
|  |  | Prostate | 0x02 | Prostate Needle Biopsy Specimen | 0x01 |
|  |  |  |  | Transurethral Resection of the Prostate Specimen | 0x02 |
|  |  |  |  | Radical Prostatectomy Specimen | 0x03 |
|  |  | Penis | 0x03 | Prepuce Biopsy Specimen | 0x01 |
|  |  |  |  | Penile Tumor Resection Specimen | 0x02 |
|  |  |  |  | Partial Penile Resection Specimen | 0x03 |
| Urinary System | 0x08 | Renal Pelvis, Ureter, Bladder, and Urethra | 0x01 | Renal Pelvis Biopsy Specimen | 0x01 |
|  |  |  |  | Renal Pelvis Resection Specimen | 0x02 |
|  |  |  |  | Renal Pelvis and Ureter Resection Specimen | 0x03 |
|  |  |  |  | Ureter Biopsy Specimen | 0x04 |
|  |  |  |  | Bladder Biopsy Specimen | 0x05 |
|  |  |  |  | Transurethral Biopsy Specimen | 0x06 |
|  |  |  |  | Partial Cystectomy Specimen | 0x07 |
|  |  |  |  | Radical Cystectomy Specimen | 0x08 |
|  |  |  |  | Radical Bladder Resection Specimen | 0x09 |
|  |  |  |  | Anterior Pelvic Organ Resection | 0x0a |
|  |  | Kidney | 0x02 | Kidney Core Needle Biopsy Specimen | 0x01 |
|  |  |  |  | Kidney Wedge Resection Specimen | 0x02 |
|  |  |  |  | Partial Nephrectomy Specimen | 0x03 |
|  |  |  |  | Radical Nephrectomy Specimen | 0x04 |
|  |  |  |  | Kidney Tumor Resection Specimen | 0x05 |
| Bone and Joint | 0x09 | Bone and Joint | 0x01 | Bone Core Needle Biopsy Specimen | 0x01 |
|  |  |  |  | Bone Curettage Biopsy Specimen | 0x02 |
|  |  |  |  | Bone Tissue Resection Specimen | 0x03 |
|  |  |  |  | Bone Marrow Cavity Margin Biopsy Specimen | 0x04 |
|  |  |  |  | Intracystic Resection Specimen | 0x05 |
|  |  |  |  | Tumor Segment Resection Specimen | 0x06 |
|  |  |  |  | Extensive Resection Specimen | 0x07 |
|  |  |  |  | Radical Resection Specimen | 0x08 |
| Soft Tissue | 0x0a | Soft Tissue | 0x01 | Soft tissue mass core needle biopsy specimen | 0x01 |
|  |  |  |  | Soft tissue mass incisional biopsy specimen | 0x02 |
|  |  |  |  | Soft tissue tumor resection specimen | 0x03 |
|  |  |  |  | Soft tissue lesion specimen | 0x04 |
| Lymphohematopoietic system | 0x0b | Lymphohematopoietic system | 0x01 | Fine needle aspiration cytology specimen | 0x01 |
|  |  |  |  | Lymph node core needle biopsy specimen | 0x02 |
|  |  |  |  | Lymph node resection specimen | 0x03 |
|  |  |  |  | Bone marrow core needle biopsy specimen | 0x04 |
|  |  |  |  | Bone marrow aspiration specimen | 0x05 |
|  |  |  |  | Spleen core needle biopsy specimen | 0x06 |
|  |  |  |  | Spleen resection specimen | 0x07 |
| Central nervous system | 0x0c | Central nervous system | 0x01 | Craniotomy resection specimen | 0x01 |
|  |  |  |  | Transnasal tissue resection specimen | 0x02 |
|  |  |  |  | Stereotactic core needle biopsy specimen | 0x03 |
|  |  |  |  | Brain lobectomy specimen | 0x04 |
|  |  |  |  | Brain tumor resection specimen | 0x05 |
| Adrenal gland | 0x0d | Adrenal gland | 0x01 | Subtotal adrenalectomy specimen | 0x01 |
|  |  |  |  | Adrenalectomy specimen | 0x02 |
| Heart and pericardium | 0x0e | Heart and pericardium | 0x01 | Complete heart excision specimen | 0x01 |
|  |  |  |  | Endocardial and myocardial biopsy specimen | 0x02 |
|  |  |  |  | Ventricular myocardium excision specimen | 0x03 |
|  |  |  |  | Papillary muscle excision specimen | 0x04 |
|  |  |  |  | Atrial appendage excision specimen | 0x05 |
|  |  |  |  | Cardiac tumor excision specimen | 0x06 |
|  |  |  |  | Heart valve excision specimen | 0x07 |
|  |  |  |  | Pericardial biopsy and excision specimen | 0x08 |
|  |  |  |  | Aortic excision specimen | 0x09 |
|  |  |  |  | Medium-sized artery excision specimen | 0x0a |
|  |  |  |  | Temporal artery biopsy specimen | 0x0b |
|  |  |  |  | Endarterectomy or atherectomy specimen | 0x0c |
|  |  |  |  | Venous excision specimen | 0x0d |
|  |  |  |  | Artificial blood vessel and stent excision specimen | 0x0e |
|  |  |  |  | Artificial valve excision specimen | 0x0f |
|  |  |  |  | Specimen Retrieval from Mechanical Circulatory Support Device | 0x10 |
|  |  |  |  | Specimen of thrombus and blood clot removal | 0x11 |
| Skin | 0x0f | Skin | 0x01 | Skin Biopsy Specimen | 0x01 |
|  |  |  |  | Skin Punch Biopsy Specimen | 0x02 |
|  |  |  |  | Skin Curettage Biopsy Specimen | 0x03 |
|  |  |  |  | Skin Excision Specimen | 0x04 |
|  |  |  |  | Skin Lesion Excision Specimen | 0x05 |
|  |  |  |  | Skin Tumor Excision Specimen | 0x06 |
| Cytology | 0x10 | Cervix | 0x01 | Liquid-Based Thin-Layer Preparation | 0x01 |
|  |  |  |  | Conventional Smear | 0x02 |
|  |  |  |  | Cell Block Slide | 0x03 |
|  |  | Thyroid | 0x02 | Liquid-Based Thin-Layer Preparation | 0x01 |
|  |  |  |  | Conventional Smear | 0x02 |
|  |  |  |  | Cell Block Slide | 0x03 |
|  |  | Pancreatobiliary System | 0x03 | Liquid-Based Thin-Layer Preparation | 0x01 |
|  |  |  |  | Conventional Smear | 0x02 |
|  |  |  |  | Cell Block Slide | 0x03 |
|  |  | Respiratory Tract | 0x04 | Lung Fine Needle Aspiration Biopsy (FNAB) Specimen | 0x01 |
|  |  |  |  | Sputum Specimen | 0x02 |
|  |  |  |  | Cell Block Slide | 0x03 |
|  |  | Serous Cavity Effusion | 0x05 | Liquid-Based Thin-Layer Preparation | 0x01 |
|  |  |  |  | Conventional Smear | 0x02 |
|  |  |  |  | Cell Block Slide | 0x03 |
|  |  | Urine | 0x06 | Specimen after urinary diversion surgery | 0x01 |
|  |  |  |  | Liquid-Based Thin-Layer Preparation | 0x02 |
|  |  |  |  | Conventional Smear | 0x03 |
|  |  |  |  | Cell Block Slide | 0x04 |

Specimen Name Sample Name

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0007 | 0x0004 | STRING | 1 |

Used to describe the specimen name of the slide, with a length not exceeding 32 bytes.

Specimen Source Specimen Source

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0007 | 0x0005 | BYTE | 1 |

Used to indicate whether the specimen for the slide was sent from another hospital. Valid values are:

0: In-house specimen

1: Outsourced specimen

Sampling Site Material Position

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0007 | 0x0006 | LONG | 1 |

DT is used to describe the slide position, represented by a <System, Specimen, Sampling Site> triplet, occupying 8 bits, 12 bits, and 12 bits respectively in a LONG data type, as shown below:

The enumeration definitions are listed in the table below:

| System Name | System ID | Specimen Name | Specimen ID | Sampling site | Sampling Site Code |
| --- | --- | --- | --- | --- | --- |
| Head and Neck | 0x01 | Thyroid and Parathyroid | 0x01 | Left lobe | 0x01 |
|  |  |  |  | Right lobe | 0x02 |
|  |  |  |  | Isthmus | 0x03 |
|  |  |  |  | Pyramidal lobe | 0x04 |
|  |  |  |  | Parathyroid gland | 0x05 |
|  |  |  |  | Lymph node | 0x06 |
|  |  | Hypopharynx (laryngopharynx), larynx, trachea, and parapharyngeal space | 0x02 | Hypopharynx (laryngopharynx) | 0x01 |
|  |  |  |  | Larynx (glottic region, supraglottic area, subglottic area) | 0x02 |
|  |  |  |  | Trachea | 0x03 |
|  |  |  |  | Pre-epiglottic space | 0x04 |
|  |  |  |  | Thyroid cartilage | 0x05 |
|  |  |  |  | Pyriform sinus | 0x06 |
|  |  |  |  | Lymph node | 0x07 |
|  |  | Nasopharynx | 0x03 | Nasopharynx | 0x01 |
|  |  |  |  | Lymph node | 0x02 |
|  |  | Nasal cavity, paranasal sinuses, and skull base | 0x04 | Nasal septum | 0x01 |
|  |  |  |  | Lateral wall of the nasal cavity | 0x02 |
|  |  |  |  | Nasal vestibule | 0x03 |
|  |  |  |  | Paranasal sinuses (maxillary sinus, ethmoid sinus, sphenoid sinus, frontal sinus) | 0x04 |
|  |  |  |  | Lymph node | 0x05 |
|  |  | Salivary gland | 0x05 | Parotid gland | 0x01 |
|  |  |  |  | Submandibular gland | 0x02 |
|  |  |  |  | Sublingual gland | 0x03 |
|  |  |  |  | Maxillofacial region | 0x04 |
|  |  |  |  | Nasal cavity | 0x05 |
|  |  |  |  | Paranasal sinuses | 0x06 |
|  |  |  |  | Oral cavity | 0x07 |
|  |  |  |  | Lymph node | 0x08 |
|  |  | Oropharynx (base of tongue, tonsil, adenoid) | 0x06 | Base of tongue | 0x01 |
|  |  |  |  | Left Tonsil | 0x02 |
|  |  |  |  | Right Tonsil | 0x03 |
|  |  |  |  | Adenoid | 0x04 |
|  |  |  |  | Lymph node | 0x05 |
|  |  | Oral cavity (lip) and mobile tongue | 0x07 | Labial Mucosa | 0x01 |
|  |  |  |  | Oral Mucosa | 0x02 |
|  |  |  |  | Tongue (lateral margin, dorsal surface, ventral surface, anterior one-third) | 0x03 |
|  |  |  |  | Gingiva (maxillary gingiva, mandibular gingiva) | 0x04 |
|  |  |  |  | Hard Palate | 0x05 |
|  |  |  |  | Buccal Mucosa | 0x06 |
|  |  |  |  | Lymph node | 0x07 |
|  |  | Odontogenic and maxillofacial bone tumors | 0x08 | Maxilla | 0x01 |
|  |  |  |  | Mandible | 0x02 |
|  |  |  |  | Lymph node | 0x03 |
|  |  | Ear | 0x09 | Left External Ear (auricle, external auditory canal, tympanic membrane) | 0x01 |
|  |  |  |  | Right External Ear (auricle, external auditory canal, tympanic membrane) | 0x02 |
|  |  |  |  | Left Middle Ear (tympanic cavity, eustachian tube, mastoid process) | 0x03 |
|  |  |  |  | Right Middle Ear (tympanic cavity, eustachian tube, mastoid process) | 0x04 |
|  |  |  |  | Left inner ear (cochlea, vestibule) | 0x05 |
|  |  |  |  | Right inner ear (cochlea, vestibule) | 0x06 |
|  |  | Eye | 0x0a | Conjunctiva and lacrimal caruncle | 0x01 |
|  |  |  |  | Iris, ciliary body, and choroid | 0x02 |
|  |  |  |  | Retina | 0x03 |
|  |  |  |  | Optic disc and optic nerve | 0x04 |
|  |  |  |  | Lacrimal gland | 0x05 |
|  |  |  |  | Lacrimal drainage system | 0x06 |
| Respiratory System | 0x02 | Lung | 0x01 | Left upper lobe of the lung | 0x01 |
|  |  |  |  | Left lower lobe of the lung | 0x02 |
|  |  |  |  | Right upper lobe of the lung | 0x03 |
|  |  |  |  | Right middle lobe of the lung | 0x04 |
|  |  |  |  | Right lower lobe of the lung | 0x05 |
|  |  |  |  | Pulmonary hilum | 0x06 |
|  |  |  |  | Bronchus | 0x07 |
|  |  |  |  | Lymph node | 0x08 |
| Pleural Tumor | 0x03 | Pleura | 0x01 | Left side | 0x01 |
|  |  |  |  | Right side | 0x02 |
|  |  |  |  | Diaphragm | 0x03 |
|  |  |  |  | Mediastinum | 0x04 |
|  |  |  |  | Pericardium | 0x05 |
|  |  |  |  | Apical Parietal Pleura | 0x06 |
|  |  |  |  | Contralateral Pleura | 0x07 |
|  |  |  |  | Visceral Pleura | 0x08 |
|  |  |  |  | Chest Wall | 0x09 |
|  |  |  |  | Rib | 0x0a |
|  |  |  |  | Incision Site | 0x0b |
|  |  |  |  | Lymph node | 0x0c |
| Digestive system | 0x04 | Esophagus | 0x01 | Upper Esophagus | 0x01 |
|  |  |  |  | Middle Esophagus | 0x02 |
|  |  |  |  | Lower Esophagus | 0x03 |
|  |  |  |  | Esophagogastric Junction | 0x04 |
|  |  |  |  | Lymph node | 0x05 |
|  |  | Stomach | 0x02 | Cardia | 0x01 |
|  |  |  |  | Gastric Fundus (Anterior Wall, Posterior Wall) | 0x02 |
|  |  |  |  | Gastric Body (Anterior Wall, Posterior Wall, Lesser Curvature, Greater Curvature) | 0x03 |
|  |  |  |  | Gastric Antrum (Anterior Wall, Posterior Wall, Lesser Curvature, Greater Curvature) | 0x04 |
|  |  |  |  | Pylorus | 0x05 |
|  |  |  |  | Lymph node | 0x06 |
|  |  | Small intestine | 0x03 | Duodenum (Bulb, Descending Part, Horizontal Part, Ascending Part) | 0x01 |
|  |  |  |  | Jejunum | 0x02 |
|  |  |  |  | Ileum | 0x03 |
|  |  |  |  | Lymph node | 0x04 |
|  |  | Colorectum | 0x04 | Colon | 0x01 |
|  |  |  |  | Rectum | 0x02 |
|  |  |  |  | Anal canal | 0x03 |
|  |  |  |  | Lymph node | 0x04 |
|  |  | Liver (including intrahepatic bile ducts) | 0x05 | Left lobe | 0x01 |
|  |  |  |  | Right lobe | 0x02 |
|  |  |  |  | Quadrate lobe | 0x03 |
|  |  |  |  | Caudate lobe | 0x04 |
|  |  |  |  | Gallbladder | 0x05 |
|  |  |  |  | Lymph node | 0x06 |
|  |  | Gallbladder and extrahepatic bile ducts | 0x06 | Gallbladder | 0x01 |
|  |  |  |  | Extrahepatic bile duct | 0x02 |
|  |  |  |  | Common bile duct | 0x03 |
|  |  |  |  | Lymph node | 0x04 |
|  |  | Pancreas | 0x07 | Pancreatic head | 0x01 |
|  |  |  |  | Uncinate process | 0x02 |
|  |  |  |  | Pancreatic body | 0x03 |
|  |  |  |  | Pancreatic tail | 0x04 |
|  |  |  |  | Gallbladder | 0x05 |
|  |  |  |  | Lymph node | 0x06 |
|  |  |  |  | Duodenum | 0x07 |
|  |  |  |  | Stomach | 0x08 |
| Breast | 0x05 | Breast | 0x01 | Left breast | 0x01 |
|  |  |  |  | Right breast | 0x02 |
|  |  |  |  | Basolateral margin | 0x03 |
|  |  |  |  | Skin lateral resection margin | 0x04 |
|  |  |  |  | Lymph node | 0x05 |
|  |  |  |  | Accessory breast | 0x06 |
|  |  |  |  | Nipple | 0x07 |
| Female Reproductive System | 0x06 | Vulva | 0x01 | Vulva | 0x01 |
|  |  |  |  | Labia majora | 0x02 |
|  |  |  |  | Labia minora | 0x03 |
|  |  |  |  | Clitoris | 0x04 |
|  |  | Vagina | 0x02 | Vagina | 0x01 |
|  |  | Cervix | 0x03 | Cervix | 0x01 |
|  |  |  |  | Cervical canal | 0x02 |
|  |  |  |  | Lymph node | 0x03 |
|  |  |  |  | Ovary | 0x04 |
|  |  |  |  | Fallopian tube | 0x05 |
|  |  | Uterine body | 0x04 | Uterine cavity | 0x01 |
|  |  |  |  | Uterus (submucosal, intramural, subserosal) | 0x02 |
|  |  |  |  | Lymph node | 0x03 |
|  |  |  |  | Ovary | 0x04 |
|  |  |  |  | Fallopian tube | 0x05 |
|  |  | Gestational trophoblastic tumor | 0x05 | Uterine cavity | 0x01 |
|  |  |  |  | Uterus | 0x02 |
|  |  |  |  | Placenta | 0x03 |
|  |  |  |  | Ovary | 0x04 |
|  |  |  |  | Fallopian tube | 0x05 |
|  |  |  |  | Lymph node | 0x06 |
|  |  | Ovary and fallopian tube | 0x06 | Left ovary | 0x01 |
|  |  |  |  | Right ovary | 0x02 |
|  |  |  |  | Left fallopian tube | 0x03 |
|  |  |  |  | Right fallopian tube | 0x04 |
|  |  |  |  | Uterus | 0x05 |
|  |  |  |  | Lymph node | 0x06 |
|  |  | Placenta | 0x07 | Placenta | 0x01 |
|  |  | Peritoneum | 0x08 | Peritoneum | 0x01 |
|  |  | Broad ligament and other uterine ligaments | 0x09 | Broad ligament of the uterus | 0x01 |
|  |  |  |  | Cardinal ligament of the uterus | 0x02 |
|  |  |  |  | Uterus round ligament | 0x03 |
|  |  |  |  | Sacrouterine ligament | 0x04 |
| Male reproductive system | 0x07 | Testis | 0x01 | Left testis | 0x01 |
|  |  |  |  | Right testis | 0x02 |
|  |  |  |  | Left epididymis | 0x03 |
|  |  |  |  | Right epididymis | 0x04 |
|  |  | Prostate | 0x02 | Left lobe of the prostate | 0x01 |
|  |  |  |  | Right lobe of the prostate | 0x02 |
|  |  |  |  | Lymph node | 0x03 |
|  |  | Penis | 0x03 | Glans penis | 0x01 |
|  |  |  |  | Corpus cavernosum of the penis | 0x02 |
|  |  |  |  | Corpus spongiosum of the urethra | 0x03 |
| Urinary System | 0x08 | Renal Pelvis, Ureter, Bladder, and Urethra | 0x01 | Left renal pelvis | 0x01 |
|  |  |  |  | Right renal pelvis | 0x02 |
|  |  |  |  | Left ureter | 0x03 |
|  |  |  |  | Right ureter | 0x04 |
|  |  |  |  | Trigone of the bladder | 0x05 |
|  |  |  |  | Right bladder wall | 0x06 |
|  |  |  |  | Left bladder wall | 0x07 |
|  |  |  |  | Anterior wall of the bladder | 0x08 |
|  |  |  |  | Posterior wall of the bladder | 0x09 |
|  |  |  |  | Apex of the bladder | 0x0a |
|  |  |  |  | Urethra | 0x0b |
|  |  |  |  | Lymph node | 0x0c |
|  |  | Kidney | 0x02 | Left kidney | 0x01 |
|  |  |  |  | Right kidney | 0x02 |
|  |  |  |  | Ureter | 0x03 |
|  |  |  |  | Adrenal gland | 0x04 |
| Bone and Joint | 0x09 | Bone and Joint | 0x01 | Cranial bone | 0x01 |
|  |  |  |  | Facial bone | 0x02 |
|  |  |  |  | Maxilla | 0x03 |
|  |  |  |  | Mandible/Temporomandibular joint | 0x04 |
|  |  |  |  | Scapula | 0x05 |
|  |  |  |  | Humerus | 0x06 |
|  |  |  |  | Radius | 0x07 |
|  |  |  |  | Ulna/Shoulder joint | 0x08 |
|  |  |  |  | Elbow joint | 0x09 |
|  |  |  |  | Carpal bones | 0x0a |
|  |  |  |  | Metacarpal bones | 0x0b |
|  |  |  |  | Phalanges/Carpal joint | 0x0c |
|  |  |  |  | Metacarpophalangeal Joint Femur | 0x0d |
|  |  |  |  | Tibia | 0x0e |
|  |  |  |  | Fibula | 0x0f |
|  |  |  |  | Patella/Hip Joint | 0x10 |
|  |  |  |  | Knee Joint | 0x11 |
|  |  |  |  | Tarsal Bones | 0x12 |
|  |  |  |  | Metatarsal bone | 0x13 |
|  |  |  |  | Phalanges/Ankle Joint | 0x14 |
|  |  |  |  | Metatarsophalangeal Joint | 0x15 |
|  |  |  |  | Clavicle | 0x16 |
|  |  |  |  | Rib | 0x17 |
|  |  |  |  | Sternum | 0x18 |
|  |  |  |  | Xiphoid Process/Sternoclavicular Joint | 0x19 |
|  |  |  |  | Cervical vertebra | 0x1a |
|  |  |  |  | Thoracic vertebra | 0x1b |
|  |  |  |  | Lumbar vertebra | 0x1c |
|  |  |  |  | Sacrum | 0x1d |
|  |  |  |  | Coccyx/Intervertebral Space | 0x1e |
|  |  |  |  | Ilium | 0x1f |
|  |  |  |  | Ischium | 0x20 |
|  |  |  |  | Pubic Symphysis/Sacroiliac Joint | 0x21 |
| Soft Tissue | 0x0a | Soft Tissue | 0x01 | Head | 0x01 |
|  |  |  |  | Neck | 0x02 |
|  |  |  |  | Face | 0x03 |
|  |  |  |  | Thoracic cavity | 0x04 |
|  |  |  |  | Abdominal cavity | 0x05 |
|  |  |  |  | Retroperitoneal space | 0x06 |
|  |  |  |  | Chest Wall | 0x07 |
|  |  |  |  | Back | 0x08 |
|  |  |  |  | Lumbar region | 0x09 |
|  |  |  |  | Axilla | 0x0a |
|  |  |  |  | Arm | 0x0b |
|  |  |  |  | Palm | 0x0c |
|  |  |  |  | Finger | 0x0d |
|  |  |  |  | Thigh | 0x0e |
|  |  |  |  | Calf | 0x0f |
|  |  |  |  | Foot | 0x10 |
|  |  |  |  | Lymph node | 0x11 |
| Lymphohematopoietic system | 0x0b | Lymphohematopoietic system | 0x01 | Cervical lymph nodes | 0x01 |
|  |  |  |  | Supraclavicular lymph nodes | 0x02 |
|  |  |  |  | Infraclavicular lymph nodes | 0x03 |
|  |  |  |  | Thoracic Lymph Nodes | 0x04 |
|  |  |  |  | Abdominal Lymph Nodes | 0x05 |
|  |  |  |  | Spleen | 0x06 |
|  |  |  |  | Axillary Lymph Nodes | 0x07 |
|  |  |  |  | Inguinal Lymph Nodes | 0x08 |
|  |  |  |  | Pelvic Lymph Nodes | 0x09 |
|  |  |  |  | Bone Marrow | 0x0a |
| Central nervous system | 0x0c | Central nervous system | 0x01 | Frontal Lobe | 0x01 |
|  |  |  |  | Parietal Lobe | 0x02 |
|  |  |  |  | Temporal Lobe | 0x03 |
|  |  |  |  | Occipital Lobe | 0x04 |
|  |  |  |  | Insular Lobe | 0x05 |
|  |  |  |  | Cerebellar Hemisphere | 0x06 |
|  |  |  |  | Cerebellar Vermis | 0x07 |
|  |  |  |  | Cerebellopontine Angle | 0x08 |
|  |  |  |  | Basal Ganglia | 0x09 |
|  |  |  |  | Thalamus | 0x0a |
|  |  |  |  | Hypothalamus | 0x0b |
|  |  |  |  | Pineal gland | 0x0c |
|  |  |  |  | Ventricles (lateral ventricles, third ventricle, fourth ventricle) | 0x0d |
|  |  |  |  | Sella region | 0x0e |
|  |  |  |  | Suprasellar region | 0x0f |
|  |  |  |  | Slope | 0x10 |
|  |  |  |  | Brainstem | 0x11 |
|  |  |  |  | Pons | 0x12 |
|  |  |  |  | Spinal cord (cervical segment, thoracic segment, lumbar segment) | 0x13 |
|  |  |  |  | Nerve root | 0x14 |
|  |  |  |  | Special locations (septum pellucidum, tectum, hippocampus, etc.) | 0x15 |
| Adrenal gland | 0x0d | Adrenal gland | 0x01 | Left adrenal gland | 0x01 |
|  |  |  |  | Right adrenal gland | 0x02 |
| Heart and pericardium | 0x0e | Heart and pericardium | 0x01 | Heart | 0x01 |
|  |  |  |  | Endocardium and myocardium | 0x02 |
|  |  |  |  | Left atrium | 0x03 |
|  |  |  |  | Right atrium | 0x04 |
|  |  |  |  | Left ventricle | 0x05 |
|  |  |  |  | Right ventricle | 0x06 |
|  |  |  |  | Papillary muscle | 0x07 |
|  |  |  |  | Left atrial appendage | 0x08 |
|  |  |  |  | Right atrial appendage | 0x09 |
|  |  |  |  | Atrial appendage | 0x0a |
|  |  |  |  | Aortic valve | 0x0b |
|  |  |  |  | Pulmonary valve | 0x0c |
|  |  |  |  | Mitral valve | 0x0d |
|  |  |  |  | Tricuspid valve | 0x0e |
|  |  |  |  | Pericardium | 0x0f |
|  |  |  |  | Aorta | 0x10 |
|  |  |  |  | Superficial temporal artery | 0x11 |
|  |  |  |  | Pulmonary artery | 0x12 |
|  |  |  |  | Coronary artery | 0x13 |
|  |  |  |  | Carotid artery | 0x14 |
|  |  |  |  | Vein | 0x15 |
|  |  |  |  | Great saphenous vein | 0x16 |
|  |  |  |  | Coronary artery | 0x17 |
|  |  |  |  | Aorta | 0x18 |
|  |  |  |  | Peripheral artery | 0x19 |
| Skin | 0x0f | Skin | 0x01 | Head | 0x01 |
|  |  |  |  | Face | 0x02 |
|  |  |  |  | Neck | 0x03 |
|  |  |  |  | Nape | 0x04 |
|  |  |  |  | Chest Wall | 0x05 |
|  |  |  |  | Back | 0x06 |
|  |  |  |  | Abdomen | 0x07 |
|  |  |  |  | Lumbar region | 0x08 |
|  |  |  |  | Buttocks | 0x09 |
|  |  |  |  | Perineum | 0x0a |
|  |  |  |  | Axilla | 0x0b |
|  |  |  |  | Arm | 0x0c |
|  |  |  |  | Palm | 0x0d |
|  |  |  |  | Dorsum of hand | 0x0e |
|  |  |  |  | Thigh | 0x0f |
|  |  |  |  | Calf | 0x10 |
|  |  |  |  | Dorsum of foot | 0x11 |
|  |  |  |  | Plantar surface | 0x12 |
| Cytology | 0x10 | Cervix | 0x01 | Cervical smear | 0x01 |
|  |  |  |  | Vaginal stump smear | 0x02 |
|  |  |  |  | Posterior fornix aspirate | 0x03 |
|  |  | Thyroid | 0x02 | Fine needle aspiration of the left thyroid lobe | 0x01 |
|  |  |  |  | Fine needle aspiration of the right thyroid lobe | 0x02 |
|  |  |  |  | Fine needle aspiration of the thyroid isthmus | 0x03 |
|  |  | Pancreatobiliary System | 0x03 | Ultrasound-guided percutaneous fine needle aspiration of the pancreas (head/neck/body/tail/body-tail) and bile duct | 0x01 |
|  |  |  |  | EUS-FNA of the pancreas (head/neck/body/tail/body-tail) and bile duct | 0x02 |
|  |  | Respiratory Tract | 0x04 | Endobronchial ultrasound-guided transbronchial needle aspiration (EBUS-TBNA) | 0x01 |
|  |  |  |  | Bronchial Brushings (BB) | 0x02 |
|  |  |  |  | Bronchial Washing (BW) | 0x03 |
|  |  |  |  | Bronchoalveolar Lavage Fluid (BAL) | 0x04 |
|  |  | Serous Cavity Effusion | 0x05 | Pleural Effusion | 0x01 |
|  |  |  |  | Peritoneal Effusion | 0x02 |
|  |  |  |  | Pericardial Effusion | 0x03 |
|  |  |  |  | Peritoneal Washing Fluid | 0x04 |
|  |  |  |  | Pelvic Washing Fluid | 0x05 |
|  |  |  |  | Testicular Hydrocele Fluid | 0x06 |
|  |  |  |  | Joint Cavity Effusion | 0x07 |
|  |  | Urine | 0x06 | Spontaneously Voided Urine | 0x01 |
|  |  |  |  | Catheterized Urine | 0x02 |
|  |  |  |  | Bladder Washing Fluid | 0x03 |
|  |  |  |  | Bladder Brushings | 0x04 |
|  |  |  |  | Renal Pelvis Brushings | 0x05 |
|  |  |  |  | Ureteral Brushings | 0x06 |

Slide Type Slide Type

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0007 | 0x0007 | BYTE | 1 |

DT is used to indicate the type of slide, with the enumeration defined as follows:

| Slide Type Name | Number |
| --- | --- |
| Blank Slide | 0x01 |
| Paraffin HE (Hematoxylin and Eosin) Slide | 0x02 |
| Frozen HE Slide | 0x03 |
| Post-Frozen HE Slide | 0x04 |
| Immunohistochemistry Slide | 0x05 |
| Special Stain Slide | 0x06 |
| Cell Block Slide | 0x07 |
| Immunofluorescence Slide | 0x08 |
| Cytological Pap Smear | 0x09 |
| Liquid-Based Thin-Layer Cytological Preparation | 0x0a |

Antibody Name Antibody Type

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0007 | 0x0008 | STRING | 1 |

Used to specify the antibody name applied to this slide, with a maximum length of 64 bytes.

Pathology Number Pathology No

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0007 | 0x0009 | STRING | 1 |

Used to specify the pathology number of this slide, with a maximum length of 32 bytes.

亚专科Subspecialty

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0008 | 0x0001 | BYTE | 1 |

DT is used to specify the subspecialty of this slide, with the enumeration defined as follows:

| Subspecialty | Subspecialty ID |
| --- | --- |
| Head and Neck | 0x01 |
| Respiratory System | 0x02 |
| Pleural Tumor | 0x03 |
| Digestive system | 0x04 |
| Breast | 0x05 |
| Female Reproductive System | 0x06 |
| Male reproductive system | 0x07 |
| Urinary System | 0x08 |
| Bone and Joint | 0x09 |
| Soft Tissue | 0x0a |
| Lymphohematopoietic system | 0x0b |
| Central nervous system | 0x0c |
| Adrenal gland | 0x0d |
| Heart and pericardium | 0x0e |
| Skin | 0x0f |
| Cytology | 0x10 |

Patient ID Patient ID

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0008 | 0x0002 | STRING | 1 |

The unique identifier of the patient in the PACS System. The same patient must use the identical Patient ID across multiple pathological sections generated from repeated pathological examinations within the same hospital.

Patient Name Patient Name

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0008 | 0x0003 | STRING | 1 |

The full name of the patient, limited to a maximum length of 64 bytes.

Patient Sex Patient Sex

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0008 | 0x0004 | BYTE | 1 |

The patient’s sex, defined by the following enumerated values:

| Sex Type | Number |
| --- | --- |
| Male | 0x01 |
| Female | 0x02 |
| Other | 0x03 |

Date of Birth Date of Birth

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0008 | 0x0005 | STRING | 1 |

The patient’s date of birth, represented as an 8-byte string in the format YYYYMMDD.

For example, 19520315 indicates the patient’s date of birth is March 15, 1952.

Card Type Card Type

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0008 | 0x0006 | BYTE | 1 |

Patient Identification Type, valid values are:

1: Identity Card

2: Hong Kong, Macau, and Taiwan Travel Permit

3: Passport

4: Military Officer Certificate

Identification Number Card No

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0008 | 0x0007 | STRING | 1 |

Patient Identification Number, length not exceeding 32 bytes.

Specimen Submission Unit Send Hospital

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0008 | 0x0008 | STRING | 1 |

Name of the specimen submission unit, length not exceeding 64 bytes.

Specimen Submission Department Send Department

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0008 | 0x0009 | STRING | 1 |

Name of the specimen submission department, length not exceeding 64 bytes.

Specimen Submission Time Send Time

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0008 | 0x000a | STRING | 1 |

Specimen submission time in the format YYYYMMDDHHMMSS.

For example, 19520315221642 indicates a submission time of 22:16:42 on March 15, 1952.

Inpatient Number Inpatient No

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0008 | 0x000b | STRING | 1 |

Patient inpatient number, length not exceeding 32 bytes.

Outpatient Number Outpatient No

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0008 | 0x000c | STRING | 1 |

Patient outpatient number, with a length not exceeding 32 bytes.

Patient Area Patient Area

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0008 | 0x000d | STRING | 1 |

The ward where the patient is located, with a length not exceeding 16 bytes.

Bed No Bed No

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0008 | 0x000e | STRING | 1 |

The bed number where the patient is located, with a length not exceeding 16 bytes.

Annotation Information Multi Annotation Sequence

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0009 | 0x0001 | SEQUENCE | 0-n |

Records the annotation information of the image data.

Rectangle Annotation Rectangle Annotation

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0009 | 0x0002 | UNDEFINED | 1 |

Used to represent rectangular annotation information for the image data; the specific definition of its value domain is as follows:

| Byte Length | Data type | Field Name | Field Description | Remarks |
| --- | --- | --- | --- | --- |
| 4 | uint32 | image id | Annotated Image ID |  |
| n | string | name | Annotation Name | UTF-8 string terminated with \0 |
| 4 | uint32 | width | Width of the rectangular bounding box |  |
| 4 | uint32 | height | Height of the rectangular frame |  |
| 4 | float | position X | X-coordinate of the rectangular frame |  |
| 4 | float | position Y | Y-coordinate of the rectangular frame |  |
| n | string | annotation text | Annotation Text | UTF-8 string terminated with \0 |

Point Annotation Position Annotation

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0009 | 0x0003 | UNDEFINED | 1 |

Used to represent point annotation information of image data, with the specific definitions of its value domain as follows:

| Byte Length | Data type | Field Name | Field Description | Remarks |
| --- | --- | --- | --- | --- |
| 4 | uint32 | image id | Annotated Image ID |  |
| n | string | name | Annotation Name | UTF-8 string terminated with \0 |
| 4 | float | position X | X Coordinate |  |
| 4 | float | position Y | Y Coordinate |  |
| n | string | annotation text | Annotation Text | UTF-8 string terminated with \0 |

Contour Annotation Outline Annotation

| MDID | EID | DT | VN |
| --- | --- | --- | --- |
| 0x0009 | 0x0004 | UNDEFINED | 1 |

Used to represent contour annotation information of image data, with the specific definitions of its value domain as follows:

| Byte Length | Data type | Field Name | Field Description | Remarks |
| --- | --- | --- | --- | --- |
| 4 | uint32 | image id | Annotated Image ID |  |
| n | string | name | Annotation Name | UTF-8 string terminated with \0 |
| 4 | uint32 | point num | Number of contour points | If the value is m (m > 0), then the next m sets of coordinates will represent all contour points. |
| 4 | float | position X | X Coordinate |  |
| 4 | float | position Y | Y Coordinate |  |
| … | … | … | … | … |
| n | string | annotation text | Annotation Text | UTF-8 string terminated with \0 |

Note: Connect all points in sequence, and connect the last point to the first point to represent the corresponding contour shape.

References

[1] Daniel C, Rojo MG, Klossa J, Della Mea V, Booker D, Beckwith BA, et al. Standardizing the use of whole slide images in digital pathology. Comput
      
      
      
      e Med Imaging Graph. 2011; 35(7-8): 496-505

[2] Pallua J D, Brunner A, Zelger B, et al. The future of pathology is digital[J]. Pathology-Research and Practice, 2020, 216(9): 153040.

[3] DICOM PS3.1 2024a - Introduction and Overview, DICOM Standards Committee

https://dicom.nema.org/medical/dicom/current/output/html/part01.html

[4] TIFF 6.0 Specifications, ITU-T, 2002

https://www.vintasoft.com/docs/resources/TIFF6-JBIG2-lossy50.pdf

[5] Markus D.Herrmann, David A.Clunie, et al. Implementing the DICOM Standard for Digital Pathology. Journal of Pathology Informatics, Volume 9, Issue 1, 2018, 37, ISSN 2153-3539.

[6] Clunie DA. DICOM Format and Protocol Standardization—A Core Requirement for Digital Pathology Success. Toxicologic Pathology. 2021;49(4):738-749.

[7] WHO Classification of Tumours Online, WHO

https://tumourclassification.iarc.who.int/home

[8] Surgical Pathology Sampling Illustrated Guide, West (USA), Wang Zhe,
      
       translated by Wang Ruian, Fourth Military Medical University Press, 2009.

[9] Application of Whole-slide Digital Image in Diagnostic Cytopathology [J], Hu Zuohong, Zhao Chun, Bao Ji, et al. Chinese Journal of Pathology, 2017, 46(8): 581–585.